Contact:  UK Agent: - Zulu Glasstek, Peter Wells     Tel 01844 201028 or 208157   http://www.zulu-glasstek.co.uk/

A lot of spares like oversize pins and GL1 wood available at http://www.ltb-krane.de/

Current AD’s available on LBA web site. http://www.lba.de/EN/DivisionT/Airworthiness/ADs/AD_node.html and more recent ADs   http://ad.easa.europa.eu/

Schleicher ADs and TN’s can be viewed at http://www.alexander-schleicher.de/en/tm-lta-wa/    All Technical notes should be reviewed to determine applicability.

Schleicher General

M	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6	https://members.gliding.co.uk/library/airworthiness/bga-inspection-042-07/

M	Elevator control			Bent elevator control arms now extended to all types with plate alloy elevator arms and

Inspection requirements clarified				BGA 004/08/2000 issue 2				5-2009

Rhönlerche II  ("Ka 4")  (Brief descriptions translated from German text. See AD for full details)

M	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

Ka-6  series   (Brief descriptions translated from German text. See AD for full details)

M	Controls				Elevator drive engagement check				BGA 003/08/2000				08/00

M	Structure			Structural inspection due at next annual after 1st Nov 2015       BGA 042/07/2004 issue 5

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

M	Flight controls			Elevator trim tab control horn end rib inspection		BGA 045/07/2005 issue 1

M	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

K6 In Service Reports (Advisory)

Wing ribs 1 & 2			inspect for cracks											03/82 & 10/80

Tail plane front mounting		damaged by carrying handle - delete handle									06/83

Elevator drive rod		damaged near elevator by misrigging									06/83

Tail plane rear mounting		cracked & worn fittings, loose & corroded bolts						    06/96, 04/94 & 06/91

Canopy catch			insecure													06/94

Trim spring & lower mounting 	bracket corroded/cracked – inspect										06/86

Airbrake paddles delaminated	bolt and glue paddles to aluminium parts									  BGA

Tail plane insecure		reinforce/repair mounting as necessary									04/87

Air brakes			Cracks in air brake aperture										02/00

Elevator trim			Split pin and washer missing from control inside fuselage

Seat				Seat repositioned causing over travel of elevator and unusual spin behaviour					5-2013

K-7

M	Rear stick 			comes out of mounting lock into place									04/83

M	Canopy 				Strap fouls rear stick/brake lever.  Fit secondary strap.

M	Controls				Elevator drive engagement check				BGA 003/08/2000				08/00

M	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

M	Flight controls			Elevator trim tab control horn end rib inspection		BGA 045/07/2005 issue 1

K7 In Service Reports (Advisory)

Fuselage centre section		cracks in welds												06/85

Rear canopies detach in flight	locating lugs removed or badly warn								02/87 & 08/91

Rudder cable 			may catch under rear airbrake pivot nut, fit guard if necessary							12/99

Air brake pivots			Pivot bolts may migrate if not locked									02/00

Jammed open in flight		Control stop jumped past stop tube										02/01

K-8  (Brief descriptions translated from German text. See AD for full details)

M	Elevator drive			Inspection of engagement					BGA 003/08/2000 Iss. 1				08/00

M	Structure			Structural inspection due at next annual after 1st Nov 2015	BGA 042/07/2004 issue 5

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

M	Flight controls			Elevator trim tab control horn end rib inspection		BGA 045/07/2005 issue 1

K8 and K18 In Service Reports (Advisory)

Wing ribs 1&2			inspect for cracks												10/80

Control system			lower mounting in fuselage centre - cracked - inspect								03/82

Seat back to be fitted (K-8)	to prevent parachute catching between tubes									11/82

Elevator hinge pins		not secure - inspect											12/86

Rudder cables			fouling elevator rod											02/92

Elevator hinge			cracked													06/92

Tail plane mountings		cracked and corroded											04/93

Fuselage tubes			X-ray inspection is possible										04/97

Elevator push rod			check for bending										 04/96 & 04/97

Elevator control (K8)		Operating bearing fitted to incorrect holes in operating arms. 							1-2007

Wings				Balsa wood packing found inside wing									4-2011

ASK-13 series (including Jubi GmbH aircraft)

M	Rear stick 			comes out of mounting lock into place									04/83

M	Canopy 				Strap fouls rear stick/brake lever.  Fit secondary strap.

M	Elevator				Inspection for bent elevator arms				BGA 004/08/2000 Iss 1				08/00

R	Controls				inspection of aileron and air brake control support brackets	BGA 008/10/2000 Iss 1				10/00

M	Elevator drive			Inspection of engagement					BGA 010/12/2000 Iss 1				12/00

M	Structure			Structural inspection due at next annual after 1st Nov 2015	BGA 042/07/2004 issue 5

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

M	Flight controls			Elevator trim tab control horn end rib inspection		BGA 045/07/2005 issue 1

ASK 13 In Service Reports (Advisory)

Canopy opened in flight		improperly locked (lack of lubrication) - colour code open/shut						02/87

Wheel brake 			lug failure												10/75

Canopy jettison			inoperative and hinge fractured - lack of lubrication								02/91

Weak link reduced to 850 Kg 	max winch/auto speed increased to 60 kts						 			12/93

Aileron balance weight supports	corroded													02/94

Airbrake drive behind rear seat	torque tube failure near drive lug weld									04/97

Air brake pivots			Pivot bolts may migrate if not locked									02/00

Rudder pedals			Failure of tube at attachment to pedal									04/00

Ballast weights			Unsecured ballast fouling controls due to bolt failure								04/00

Landing Gear			Suspension “Donuts” displaced after heavy landing and restricting controls					08/02

Spin training			In extended weight category, K13’s are approved for spin training whilst operating. Other aerobatics prohibited.	1-2007

Air Brake Control		Rear Air Brake lever broken at rod attachment bearing location					6-2008, 5-2010

Aileron. Airbrake			Cracked bracket at wing root (BGA Inspection 008/10/2000 refers)						5-2010

Battery Box			Battery secured with cable ties due to broken support								1-2011

Nose Wheels			Non Schleicher conversions susceptible to damage from hard landings						1-2011

Seat Harness			Incorrect bolt fitted to harness anchor point									2-2013

Rudder pedals			Ensure fwd rudder cables are correct length to avoid pedals fouling nose cone					3-2013

Lower sternpost                                Rotten wood at lower sternpost and extremely corroded bolts                                                                                          2-2015

ASK-14 also see general and airbrake sections

M	Controls				Elevator tube spar inspection				AD 70-86, TN 3

M	Controls				Cable inspections Nicropress sleeves			AD 82-216

M	Fuel system			Fuel tank						AD 89-121, TN 6					10/89

M	Controls				L’Hotellier connectors					AD 1993-001/3

R	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

ASK 16

R	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

ASK 16 In Service Reports (Advisory)

Control tubes in the wing		corroded													02/92

ASK 18

M	Controls				Elevator drive engagement check				BGA 010/12/2000				12/00

M	Structure			Glue inspection every 3 years at annual after 1st Nov 2015	BGA 042/07/2004 issue 6

M	Flight controls			Elevator rib 1 inspection increased frequency to every year	BGA 043/07/2004 issue 2				1-2015

M	Rudder control			Inspection & modification of rudder cable guide tube 		BGA 026/03/2002 Issue 1

M	Flight controls			Elevator trim tab control horn end rib inspection		BGA 045/07/2005 issue 1

K8 and K18 In Service Reports (Advisory)

Wing ribs 1&2			inspect for cracks												10/80

Control system			lower mounting in fuselage centre - cracked - inspect								03/82

Seat back to be fitted (K-8)	to prevent parachute catching between tubes									11/82

Elevator hinge pins		not secure - inspect											12/86

Rudder cables			fouling elevator rod											02/92

Elevator hinge			cracked													06/92

Tail plane mountings		cracked and corroded											04/93

Fuselage tubes			X-ray inspection is possible										04/97

Elevator push rod			check for bending										 04/96 & 04/97

Elevator control (K8)		Operating bearing fitted to incorrect holes in operating arms. 							1-2007

Structures			Delamination of spar ply found during 5 year inspection							2-2011

Schleicher  Wood / Metal general - In Service Reports (Advisory)

Flying controls			Aileron horn and balance weight support are corroded							04/05

Wheel brake cables 		to be wire wiped at turnbuckle to prevent air brake restriction							10/66

Wings, damage 			by water/damp ply separation & spar damage					      02/00, 10/83, 08/83 & 08/76

Rudder pedals (front) 		cracked and bent - clips broken								 08/94 , 06/84 & 02/77

Rudder hinges (top & bottom)	cracked										      04/87, 02/84, 04/80 & 01/79

Rudder mounting brackets		cracked												04/91 & 02/90

Lower rudder hinge mount 	cracked	repair and reinforce with 2mm ply as necessary							06/97

Elevator drive (not Ka-6E & 14)	check full engagement after repair										06/87

Rigging of ailerons		both neutral with elevator neutral										08/87

Fin attachments	 (rear)		damage by water and corroded		 					       08/98, 06/95, 01/93 & 02/90

Steel tubes around wheel boxes	rusted through - lack of drainage										04/90

Trim tab root rib			glued joint failure	 tab detached - introduce drain holes							08/90

Rear fuselages (ASK 13 & K 7)	structural failures												02/91

Control connectors at wing root	bending of thread - adjust correctly/fit airbrake system stop

half connector top may be cut away to give clearance with bellcrank						02/91

Trim system turnbuckles		failure at trim lever											08/91

Fin attachments			ply cracked (front) & rotten at bottom								08/87 & 06/92

Canopy locks worn Ka 2/2b (& Ka 7?)	canopies open during sideslips									08/97

Tail plane mounting bolt		threads wear - replace bolts as necessary									06/99

Rudder Pedal Failure Ka13	Reported failure.											04/00

Unsecured ballast weight 		Attachment bolt failed											04/00

Bogus parts			Problems with bogus parts											04/00

Fuselage fittings			K7 low wing conversion found with excessive material removed from wing attachment lugs			4/2006

Control connections		K13 (applicable to other types) Aileron disconnect safety/locking pin bent and missed rigging pin hole

Replace safety pins if bent or distorted									4/2006

Elevator control (K8)		Operating bearing fitted to incorrect holes in operating arms. Could also affect other similar types			1-2007

Structure			Delaminated stern post after hard ground strike								2-2007

Rudder pedals			Saddle clamps broken causing pedals to jam on fuselage frame							2-2007

Wheels				Broken tie bolts. Take care when removing wheels, deflate if necessary						2-2007

Airbrake stops insecure		rivet them into place											07/76

Cross-link in centre section 	(not ASK-13) cracked and twisted - reinforce						  04/89, 12/88 & 12/84

Airbrake handle failures		cracks at rivet and bearing holes							       10/95, 04/94, 06/92 & 10/88

Airbrake paddles (lower)		delamination, lack of drain holes									04/93 & 04/89

Airbrakes  (Ka-2B & K-7)		reduction of travel and loads									02/85 & 08/89

recommended airbrake travel 	K-7, restriction to prevent heavy landings Paddles should not quite clear cut-outs in wings.			08/99

Jammed open in flight		Operating arm attach. Bolt to paddle fouling wing box. Incorrectly positioned slot				02/01

ASK 13 Airbrakes		Continuing problems with cracked “A” brackets at wing root – see pervious					08/03

Controls	 (K6cr and poss others)	Aileron disconnect due QR safety clip fouling ctr. Section shelf. Ensure only correct size clip is used		10/03

Controls	 (K13 and others)		damage to control rods in ctr. Section due to rigging damage. Check for bent rods and fittings.			10/03

Fin attachment			It is recommended to pack lower face of fin against fuselage tubes to prevent flexing				04/04